HEC-RAS			Chapter 3 of the Users Manual

CIEG 440 Water Resources Engineering

- Start,  click on HEC-RAS in the programs folder.

- Start new project

- File, then select new project, call it WCC at Wedgewood Road footbridge.

- Enter geometric data.

- Select geometric data from Edit menu.

- Press river reach button and draw in reach from upstream to downstream.

- Enter a river and reach identifier. For cross section station click on RS

- Enter crosse section/hydraulic structure data.  Press cross section button.  River station =  0+00, 1+15 etc. Cross sections numbered proceeding upstream.

- Save file.  File, then Save Geometric Data as.

- Enter downstream flow water level.

- Edit steady flow.  Steady flow data editor.  Insert flows.

- Reach boundary conditions.  Starting water surface elevation.

- File, then Save flow data as from Steady Flow Data menu bar.

- Perform hydraulic calculations.

- Run menu bar option.  Go to steady flow.

- File menu of steady flow analysis.  Select new plan.

- Subcritical flow regime.

- Press compute button at bottom of steady flow analysis window.

- View results.

- Go to view options.

- Select cross section, profile, 3D plots.

- Select table summaries.